| RELEASE VERSION: | 1.5 |  | PREVIOUS RELEASE VERSION: |  |  | 1.4 |
| --- | --- | --- | --- | --- | --- | --- |
| FILE NAME: | RD10031-fpga-1.5.08006F26.pof |  | SOFTWARE TOOL |  |  |  |
| FILE FORMAT(S): | Programmer Object File (.pof) |  | Quartus Prime Programmer |  |  |  |
| MEDIA TYPE: | MAX 10 FPGA U4 |  | N/A |  |  |  |
| CHECKSUM: | RD10031-fpga-1.5.08006F26.manifest.txt |  | FILE SIZE (number of byte): |  |  | 84232 (.pof) |
| RELEASE TYPE: | PRODUCTION [X] |  |  | INTERIM [ ] |  |  |
| SPECIAL INSTRUCTIONS: | .pof: Program U4 with the file content. Use Intel FPGA Download Cable II (via J17). |  |  |  |  |  |
| MANUAL REQUIRED (PART#): |  |  |  |  |  |  |
| HARDWARE REQUIREMENTS INCLUDED | YES [] NO [X] | BIOS DETAILS INCLUDED |  |  | YES [] NO [X] |  |
| SOFTWARE OPTIONS/ USER INTERFACE INCLUDED | YES [] NO [X] | LIST OF MODIFICATIONS INCLUDED |  |  | YES [X] NO [] |  |
| TESTS INCLUDED | YES [] NO [X] | LIMITATION, BUG / GENERAL INFO INCLUDED |  |  | YES [] NO [X] |  |
|  | DATE | SIGNATURE |  |  |  |  |
| Originator: |  | Simon Shink |  |  |  |  |
| Engineering Manager: |  | Alexandre Marcoux |  |  |  |  |
| Production Test: |  | Ziqiang Chen |  |  |  |  |

Source Control Tags

| File generated | Source Control Software | Project name | Tag / Label name |
| --- | --- | --- | --- |
| RD10031-fpga-1.5.08006F26.pof | Mercurial | hwfpga: projets\RD10031\fpga\prj\RD10031.qpf | RD10031-fpga-1.5.0 |

# Modifications:

- Disconnect LOAD_SHED_N from MXM1_PWR_LEVEL <= '1'; MXM2_PWR_LEVEL <= '1'; To keep MXM performance even under 9v power feed voltage.

- Disconnect LOAD_SHED_N from THRM_N to keep cpu performance under 9v.

- Add LOAD_SHED_N status in register 0x1 bit 6

- Modify fpga doc

TESTS:

| Equipment |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Board: | N/A |  | Serial number: | N/A |  |
| DRAM: | N/A |  | BMC | N/A |  |
| IDE: | N/A |  | BIOS | N/A |  |
| CPU | N/A |  |  |  |  |
| Test |  | Description |  |  | Result |
| Test bench |  | FPGA project test bench |  |  | Pass |
| Automated tests |  | FPGA project test suite |  |  | N/A |